(Run macros in order of sheets – 2-4)

Download the data from ftp://ftp.atdd.noaa.gov/pub/GEWEX/2010/ch/

Change the year in the link to get the data of that year

Get the data for the year that you want to process

Convert it into columns in excel

IF the datime (6:00-19:00) the number is greater than 5000, take the average of the cells above and below.

In the night time, set the number to 0.

Add the values of latent heat to get value for each day.

Mutilply the w/m2 latent heat by 0.000798 to get mm/day ET

To get the temperature in C for each hour, take the average of Air temperature.

To get the wind speed in m/s for each hour, take the average and multiply by 0.410631.

To get the net radiation in MJ/mh for each hour, add the net radiation and multiply by 0.0036.

To get G in MJ/mh for each hour, add the net radiation and multiply by 0.0036 * 0.04

To get the incoming radiation in w/m2 for each hour, take the average of incoming radiation.

To get the relative humidity for each hour, take the average of relative humidity.

To get Ea, use the formula

Ea = (relhum / 200) * 0.6108 * Exp((17.27 * (temp / 2)) / ((temp / 2) + 237.3))

Then Td can be calculated from

Td = (237.3 * [Log (Ea)/0.6108]) / (17.27 - [Log (Ea)/0.6108])

Refer to the Equation 1 in the link - http://www.decatur.de/javascript/dew/about.html#Equation1

Then with these values calculate the daily ETr using the unPMhr sheet

Take the Hourly Etr from unPMhr sheet and add for each day to get the daily Etr

Find the satellite over pass time for ChestnutRidge from the below link

http://rapidfire.sci.gsfc.nasa.gov/realtime/?calendar

Convert the hourly temperature from C to F

Convert the hourly windspeed from m/s to mph

Multiply the hourly incoming radiation in w/m2 with 0.086 to get hourly Solar radiation in L/hr

Now pick up the Daily Etr and Relative humidity, Windspeed, Solar Radiation and temperature for the satellite overpass time. Submit this data into the RSET link to get the values http://rs.tnstate.edu/rset/